TECHNICAL REQUIREMENT FOR

PCB FABRICATION, COMPONENTS PROCUREMENT AND ASSEMBLY OF ELECTRONIC MODULES

Doc No: MCSRDC/HJT36/MC4/HW/FAB-ASSY/01/00

Date:13th May, 2025

Hindustan Aeronautics Limited

Mission and Combat System Research and Design Centre

Bangalore – 560037

This document is the Property of Hindustan aeronautics Ltd. This document should not be reproduced or communicated without written authorization.

TECHNICAL REQUIREMENT FOR

PCB FABRICATION, COMPONENTS PROCUREMENT AND ASSEMBLY OF ELECTRONIC MODULES

Doc No: MCSRDC/HJT36/MC4/HW/FAB-ASSY/01/00

Date:13th May, 2025

TABLE OF CONTENTS

1.	Purpose	4

2.	Scope of Work	4

3.	PCB Board Specifications:	5

4.	Detailed Requirement:	6

4.1	Stage 1 – Fabrication Detailed Requirement:	6

4.1.1	General Requirements of PCB Fabrication	6

4.1.2	Input from MCSRDC, HAL	6

4.2	Stage 2 –Component Procurement and Verification Detailed Requirement:	7

4.3	Stage 3 – PCB Assembly Detailed Requirement:	7

4.3.1	General Requirements of the PCB Component Assembly	7

4.3.2	Input from MCSRDC, HAL	8

4.4	Stage 4 – Conformal Coating Detailed Requirement:	8

4.4.1	Conformal Coating requirements:	8

4.4.2	Conformal Coating instructions:	9

4.4.3	Input from MCSRDC, HAL	9

4.4.4	Component Rework requirements:	9

4.4.5	Component Rework instructions:	10

4.4.6	Input from MCSRDC, HAL	10

5	Other Requirements	10

6	List of Jobs:	11

7	Deliverables	11

7.1	PCB Fabrication Deliverables	11

7.2	Component Procurement and Verification Deliverables	11

7.3	PCB Component Assembly Deliverables	12

7.4	PCB Conformal Coating Deliverables	12

7.5	PCB Rework Deliverables	12

8	Delivery Schedule	12

9	Proposal	12

10	Acceptance	14

11	Repeat Order Requirement	14

12	Warranty	14

- Purpose

Purpose of this document is to provide technical requirement of 4 types of electronic cards (i.e. IOCM, DIOM, Backplane and SPMC) for PCB fabrication, Component Procurement & Verification, Component Assembly, Conformal Coating and Rework by the vendor along with quality process reports.

- Scope of Work

Vendor shall be responsible for MIL Grade PCB fabrication, Component Procurement & Verification, Component Assembly, Conformal Coating and Rework along with providing the reports for IOCM, DIOM, Backplane and SPMC cards as detailed in further section of this document. A total of 8 No. of boards for each type of cards are to be fabricated, assembled and supplied to HAL. Also vendor shall procure additional  2 sets of spare components for each type of cards and supply to HAL.

SoW is divided into 3 stages as mentioned below.

Stage-1: PCB Fabrication

Fabrication of 8 No. MIL grade PCBs for IOCM, DIOM, Backplane and SPMC cards each.

Stage-2: Component Procurement and Verification

Components for 8 No. of boards along with additional 2set spare components (total 10 set as per BOM attached in annexure-1, 2 and 3) for IOCM, Backplane and SPMC cards shall be procured by vendor as per the Bill of materials (BOM) provided by HAL and kit binned in co-ordination with HAL MCSRDC and ORDAQA.

Stage-3: Component Assembly

Component Assembly on MIL grade PCB for IOCM, DIOM, Backplane and SPMC cards.

Vendor shall assemble Four types of Boards. Assembly of each type of Board shall be done in a maximum of 5 batches. Each batch can have minimum 1 No of Board and Maximum 8 No of Boards. Assembly of one Board is considered as one Assembly Job. Same notation will be used from now on in this document. Assembly shall be done as per the assembly guidelines provided by MCSRDC, HAL. Vendor shall also generate necessary assembly reports that are required by certifying agencies. DIOM components will be provided by HAL after kit clearance from certification agency.

Stage-4: Conformal Coating

Conformal coating on MIL grade PCB for IOCM, DIOM, Backplane and SPMC cards.

Vendor shall do Conformal Coating on Four types of boards. Conformal Coating of one Board is considered as one Conformal Coating Job. Same notation will be used from now on in this document. Conformal Coating of each type of board shall be done in a maximum of 5 batches. Each batch can have minimum 1 No of Board and Maximum 8 No of Boards.

Stage-5: Assembly Rework

Assembly Rework on MIL grade PCB for IOCM, DIOM, Backplane and SPMC cards.

Vendor shall do rework on Four types of boards. Rework of one Board is considered as one Rework Job. Same notation will be used from now on in this document. Rework of each type of board shall be done in a maximum of 5 batches. Each batch can have minimum 1 No of Board and Maximum 8 No of Boards.

- PCB Board Specifications:

  - IOCM

| PCB dimension | :233.35 mm (L) x 160 mm (W) |
| --- | --- |
| Thickness of Board | :2.2 mm – 2.4 mm |
| PCB Material | : Glass Epoxy-FR4 (FR4-370HR) |
| Fabrication Standard | : As per MIL-PRF-31032 |
| Type | : Strip line Design |
| No. of Layers | :14-24 layers |
| Surface Finish | : ENIG |
| BOM details | : Annexure-1 |

  - DIOM

| PCB dimension | :233.35 mm (L) x 160 mm (W) |
| --- | --- |
| Thickness of Board | :2.2 mm – 2.4 mm |
| PCB Material | :Glass Epoxy-FR4 (FR4-370HR) |
| Fabrication Standard | :As per MIL-PRF-31032 |
| Type | : Strip line Design |
| No. of Layers | :16-24 layers |
| Surface Finish | : ENIG |
| BOM details | : Annexure-2 |

  - Backplane

| PCB dimension | : Refer Annexure:4 - Dimension Details of Backplane & Connector Block PCB with Flexi |
| --- | --- |
| Thickness of Board | :3.6±0.2 mm |
| PCB Material | : Glass Epoxy-FR4 for Rigid Part and Polyamide (Krypton) for Flexi Part |
| Fabrication Standard | : As per MIL-PRF-31032 |
| Type | : Strip line Design |
| No. of Layers | :20 to 24 layers of Rigid and 16 to 20 for flexi |
| Surface Finish | : ENIG with gold plating |

  - SPMC

| PCB dimension | :143.75 mm (L) x 74 mm (W) |
| --- | --- |
| Thickness of Board | :1.93 ± 0.1 mm |
| PCB Material | : Glass Epoxy-FR4 (FR4-370HR) |
| Fabrication Standard | : As per MIL-PRF-31032 |
| Type | : Strip line Design |
| No. of Layers | :12-16 layers |
| Surface Finish | : ENIG |
| BOM details | : Annexure-3 |

- Detailed Requirement:

PCB fabrication, Component Procurement & Verification (As mentioned in the document Section No. 2 and 3), Component Assembly, Conformal Coating and Rework of PCBs shall be done by the vendor based on the Gerber file, Bill of Materials (BOM) and other details provided by HAL after the placement of PO.

  - Stage 1 – Fabrication Detailed Requirement:

PCB fabrication for all boards shall be of MIL Grade (8 quantities of each type) and shall be done by the vendor by using Gerber file details provided by HAL, MCSRDC after placement of PO. All PCBs shall be impedance controlled. Base Copper Thickness shall be 35 microns or 18 microns as per the input provided by HAL.

IOCM, DIOM and SPMC Card were fabricated through M/s Micropack, Bengaluru and Backplane through M/s Cimulec France. However, vendor may go for fabrication of PCBs with the company having approval for the MIL grade PCB Fabrication, Quality Management System (ISO 9001) or Aerospace AS9100 certification/approval and shall provide Group-A, Group-B certificate of PCBs. PCB fabrication shall be done as per MIL-PRF-31032 standard.

    - General Requirements of PCB Fabrication

PCB Fabrication shall ensure the following points:

- Vendor shall provide a copy of MIL grade PCB Fabrication and Quality Management System (ISO 9001) or Aerospace AS9100 certification/approval, MIL-PRF-31032 certificate of selected PCB fabrication vendor along with the technical proposal.

- Fabrication shall be as per the specifications of the of PCBs mentioned in section-3. Gerber files of PCBs shall be provided after PO Placement.

- The inputs provided by MCSRDC, HAL are restricted between HAL and Vendor; therefore, Vendor shall not share these details to any other third party without written permission from HAL.

- NRE, the Setup charges for each type of PCB required for PCB fabrication to be quoted separately.

- Inspection of fabricated Bare PCBs shall be done by vendor. Fabricated bare PCB power supplies impedance checks to be done by the vendor.

- Group A and Group B certificate shall be provided for each IOCM, DIOM, Backplane and SPMC board.

- Vendor shall provide the COC for the fabricated PCBs.

    - Input from MCSRDC, HAL

- Gerber file details of all four types of boards.

  - Stage 2 –Component Procurement and Verification Detailed Requirement:

- Component procurement and Verification for 8 no. of cards along with additional 2 sets of components for each type of cards (Total 10 Set for IOCM, Backplane and SPMC) shall be the responsibility of the vendor. Component procurement shall be as per the BOM attached in Annexure-1, 2 & 3.

- All components procured shall be of MIL grade or industrial grade as per the part number mentioned for each component in the verified BOM.

- Components shall be procured from the OEM/OEM Authorized distributor only. In case of any deviation, the concurrence from MCSRDC, HAL shall be taken before procurement.

- All the components shall have latest date codes.

- Vendor shall get AS 6081 certificate and solderability test report for obsolete IC’s in case of unavailability of OEM CoC. Refer notes in Annexure-1, 2 & 3.

- Vendor may suggest alternate part for passive components in case particular P/N is obsolete. Refer notes in Annexure-1, 2 & 3.

- Vendor shall provide the OEM/ OEM Authorized distributor COCs for all the procured components.

- The Inward material inspection report, hardcopy and softcopy (PDF format) of manufacturer’s Certificate of Compliance (COC) for the components procured shall be provided by the vendor to HAL, MCSRDC

- Vendor shall perform kit-binning in co-ordination with certification agencies namely HAL-MCSRDC- Quality group, CEMILAC and ORDAQA and generate the required reports (inward inspection report, kit binning report as per AQA directives, observation closure etc.) at vendors place.

  - Stage 3 – PCB Assembly Detailed Requirement:

PCB Assembly of 8 numbers of MIL grade boards for IOCM, DIOM, Backplane and SPMC cards shall be done with HAL verified & approved stage1 and stage2 outputs. DIOM components will be provided by HAL after kit clearance from certification agency.

    - General Requirements of the PCB Component Assembly

PCB Component Assembly shall ensure the following points:

  - Vendor identified PCB Assembly house should have the following certification for the PCB Assembly and manufacture job and required to submit the copy of the same along with the proposal.

- Vendor identified PCB Assembly house shall have all the facilities for the ESD protection required for the electronic components and Environmental Management System (ISO 14001) certification.

- Vendor identified PCB Assembly house shall have valid Quality Management System (ISO 9001) or Aerospace AS9100 certification, IPC membership/ approval/certification.

  - Assembly processes shall be RoHS / non- RoHS compliant based on the respective components and shall be compliant with IPC-standard.

  - The HAL Quality Control group and Certification agency may visit the place of assembly of vendor as per the requirement.

  - Mounting guidelines and soldering profile instructions need to be taken care for all components from respective datasheets/OEM Guidelines.

  - Vendor shall follow the assembly instruction and assembly guidelines as per J-STD-020 & data sheet of the components.

  - Baking of the PCBs & components shall be performed before assembly as per J-STD-033 & the baking instructions provided by the component OEM datasheet.

  - ESD safe soldering station for through-hole components and for wiring shall be used.

  - The workmanship should be as per the IPC-A-610D electronic assembly standard.

  - Impedance checks at power supply reference points of the assembled board shall be done by vendor after the assembly at vendor place and shall provide the report to HAL.

  - Inspection of assembled PCBs shall be done by vendor in accordance with the ATP prepared by HAL, MCSRDC.

  - Any assembly error found during the acceptance checks will be brought to the notice of the vendor and the same shall be corrected by the vendor without any extra charge.

  - X-Ray report for the assembled Boards and reworked Boards for BGA, LGA, QFN and for the components whose pins/ balls are not visible from outside shall be required.

  - Vendor shall take the complete responsibility of rework in case of any issues identified in X-Ray report or any other issue related to assembly process.

  - NRE, the Setup charges for each type of PCB required for PCB Assembly to be quoted separately.

    - Input from MCSRDC, HAL

- Gerber file details of all four types of boards.

- XY location details of all four types of boards.

- PCB Assembly drawings of all four types of boards.

- BOM of all four types of boards.

- Bare PCBs with HAL material Gate Pass for component Assembly and Return

- Components for DIOM after kit clearance by HAL

  - Stage 4 – Conformal Coating Detailed Requirement:

    - Conformal Coating requirements:

Conformal Coating of Four types of electronic boards shall be done by the vendor. Each Type of Electronic Board shall be considered as separate Batch. For each type of Electronic Board 8 Conformal Coating Jobs are required. Maximum of 5 Batches for each Type of Electronic Board are required. Required Boards will be provided by HAL.

    - Conformal Coating instructions:

- Conformal Coating shall be done using standard practices used for Military Grade Electronic Boards.

- Vendor shall do necessary cleaning and Drying of Boards before Conformal Coating

- Vendor shall do necessary masking on connectors and other locations as per the Masking Instructions provided by HAL.

- Vendor shall use MIL-Grade Humiseal 1B31 for Conformal Coating.

- Baking of Conformal Coated Boards shall be done as per instructions provided by HAL.

- Vendor shall arrange to collect the boards from MCSRDC and return them to MCSRDC after completion of conformal coating.

    - Input from MCSRDC, HAL

- Masking Instructions for Conformal Coating.

- Instructions for Conformal Coating and Baking.

- Boards for Conformal Coating.

Stage 5 – Rework Jobs Detailed Requirement:

    - Component Rework requirements:

Component rework of four types of electronic boards shall be done by the vendor whenever there was a failure found in the board. Each Type of Electronic Board shall be considered as separate Batch. For each type of Electronic Board 5 Rework Jobs are required. Maximum of 5 Batches for each Type of Electronic Board are required. Required Boards & Components for Rework Jobs will be provided by HAL.

For each Rework Job Maximum of 4 No’s of any package (BGA Components, QFN Components, Components with Thermal Pads, SMD Components, Through Hole Component and etc.) will be required to be reworked on Board. Each Rework Job may have different type of components; therefore, vendor shall consider this.

Rework Case1: Rework shall include removal of old component and assembly of new component (New Component will be provided by HAL). Therefore, in this case One Rework Job means maximum of 4 No’s of old Component’s removal and 4 No’s of new Components Assembly in One Board.

Rework Case2: Rework shall include removal of old component from one board and assembly of removed component on second board. Therefore, in this case One Rework Job means maximum of 4 No’s of Components removal from one board and 4 No’s of removed Components Assembly on Second Board.

Rework Case3: Rework shall include assembly of new component on board where old components were already removed. Therefore, in this case One Rework Job means Assembly of 4 No’s of new Components Assembly on Board.

    - Component Rework instructions:

- Rework processes shall be RoHS / non- RoHS compliant based on the      component.

- Rework shall be compliant with IPC-standard.

- Vendor shall follow the Rework guidelines as per HAL recommendations & data sheet of the components.

- Local removal of Conformal Coating shall be done before Rework (if Board is already Conformal Coated). Also, local conformal coating shall be done on removed place as per HAL instructions.

- Baking of the components shall be performed before assembly as per J-STD-033 & the baking instructions provided by the component OEM datasheet.

- Vendor shall arrange to collect the boards from MCSRDC and return them to MCSRDC after completion of conformal coating.

    - Input from MCSRDC, HAL

- Gerber file details of required board for rework.

- XY location details of all four types of required board for rework.

- PCB Assembly drawings of required board for rework.

- BOM of required board for rework.

- Boards & New Components of required board for rework.

- Other Requirements

- NRE, the Setup charges for each type of PCB required for PCB fabrication and Assembly to be quoted separately.

- If there is no change in HAL provided inputs from one batch of Assembly Job to other, Batch Stencil & Setup charges shall not be considered for that particular batch of Assembly Job. However, Batch Stencil & Setup charges shall be quoted separately in price bid.

- Collection of delivered PCBs and delivery of assembled boards/reworked boards/conformal coated boards shall be in vendor scope.

- The returnable inputs (If any) provided by HAL, MCSRDC shall be returned back, after the execution of work. Components provided by HAL or procured by the vendor as per the attached BOM which are not used in the assembly of the respective boards shall be returned back to HAL.

- MSL Levels and Baking hours information for all the components and the PCBs needs to be taken care by the vendor.

- Vendor shall provide proof of similar jobs done earlier at the time of proposal.

- List of Jobs:

- IOCM PCB fabrication with Reports, Qty: 8nos

- DIOM PCB fabrication with Reports, Qty: 8nos

- Backplane PCB fabrication with Reports, Qty: 8nos

- SPMC PCB fabrication with Reports, Qty: 8nos

- IOCM components procurement with CoC & Reports, Qty: 10nos

- Backplane components procurement with CoC & Reports, Qty: 10nos

- SPMC components procurement with CoC & Reports, Qty: 10nos

- IOCM Assembly Jobs along with Assembly Reports, Qty: 8nos

- DIOM Assembly Jobs along with Assembly Reports, Qty: 8nos

- Backplane Assembly Jobs along with Assembly Reports, Qty: 8nos

- SPMC Assembly Jobs along with Assembly Reports, Qty: 8nos

- IOCM Conformal Coating Jobs along with Conformal Coating Reports, Qty: 8nos

- DIOM Conformal Coating Jobs along with Conformal Coating Reports, Qty: 8nos

- Backplane Conformal Coating Jobs along with Conformal Coating Reports, Qty: 8nos

- SPMC Conformal Coating Jobs along with Conformal Coating Reports, Qty: 8nos

- IOCM Rework Jobs along with Rework Reports, Qty: 5nos

- DIOM Rework Jobs along with Rework Reports, Qty: 5nos

- Backplane Rework Jobs along with Rework Reports, Qty: 5nos

- SPMC Rework Jobs along with Rework Reports, Qty: 5nos

Note: These Jobs shall be executed within a span of 3 Years from the date of PO.

- Deliverables

  - PCB Fabrication Deliverables

The below items are to be delivered after PCB Fabrication. One set of hard copy and soft copy of below items shall be delivered for PCBs as mentioned in section 3.

  - Fabricated PCB along with Group A certificates.

  - Manufacturer COC of the fabricated PCB.

  - Component Procurement and Verification Deliverables

The following deliverables are applicable as mentioned in section 3.

- Group B certificates of Fabricated PCBs (Hard copy & Soft copy).

- Kit binning for 10 Set (8Set + 2Set) of procured component as per BOM including 2 Spare Set for IOCM, Backplane and SPMC. The ORDAQA cleared kit binning reports and memo shall be considered as deliverable for these 8 sets. The spare 2 sets of components shall be delivered directly to MCSRDC.

- OEM/ OEM Authorized distributor COCs for all the procured components-Hardcopy.

- Vendor shall get AS 6081 certificate and solderability test report for obsolete IC’s in case of unavailability of OEM CoC.

- Inward component inspection report, kit binning report for all the components (Hard copy & Soft copy).

  - PCB Component Assembly Deliverables

The following deliverables are applicable for 8 No. of cards for each type as mentioned in section 3.

- 8 Set of assembled boards for each type (IOCM, DIOM, Backplane and SPMC) with kit binned components available with vendor from Section 7.2 and DIOM Components provided by HAL.

- Component Baking Report. (Hardcopy)

- Assembly Inspection report (mounted, DNM). (Hardcopy)

- X-Ray report for the assembled PCBs for BGA, LGA, QFN and for the components whose pins/ balls are not visible from outside shall be required. (Softcopy-In CD)

- Quality check report for the assembly. (Hardcopy)

- All stencils and thermal profiles to be supplied after execution of PO.

  - PCB Conformal Coating Deliverables

Conformal Coated Boards along with Conformal Coating Reports.

  - PCB Rework Deliverables

Reworked Boards along with Rework & X-Ray Reports.

- Delivery Schedule

| S.N. | Stage | Deliverables | Qty | Delivery Schedule |
| --- | --- | --- | --- | --- |
| 1 | Stage-1 | Deliverables as mentioned in section 7.1 for PCB fabrication | 8 Set for each type | Within T1+ 8 weeks |
| 2 | Stage-2 | Deliverables as mentioned in Section 7.2 for IOCM, Backplane and SPMC | 10 Set for each type | Within T1+ 20 weeks |
| 3 | Stage-3 | Deliverables as mentioned in Section 7.3 for Assembly | 8 Set for each type | Within T2 + 2weeks |
| 4 | Stage-4 | Deliverables as mentioned in Section 7.4 for Conformal coating | 8 Set for each type | Within T3 + 1 weeks |
| 5 | Stage-5 | Deliverables as mentioned in Section 7.5 for PCB Rework | 5 Set for each type | Within T4 + 2 weeks |

Note:

T0:  Time for Placement of PO

T1:  Date of final inputs from MCSRDC (T1 will not be more than 2 weeks from T0)

T2: Date of final inputs from MCSRDC for Assembly. Vendor shall collect required boards from MCSRDC, HAL, Bangalore for assembly and deliver back.

T3: Date of final inputs from MCSRDC for Conformal coating. Vendor shall collect required Boards from MCSRDC, HAL, Bangalore and do conformal coating and deliver back.

T4: Date of final inputs from MCSRDC for Rework. Vendor shall collect required Components and Boards from MCSRDC, HAL, Bangalore, do rework and deliver back.

- Proposal

Vendor shall provide compliance against each column in the RFQ. Vendor shall provide costing separately for each line item that are mentioned below.

| SN. | JOB | Qty. |
| --- | --- | --- |
|  | NRE for fabrication of MIL Grade IOCM PCB | 1 |
|  | Fabricated MIL Grade IOCM PCB along with Group-A report and CoC | 8 |
|  | Group-B Report for Fabricated MIL Grade IOCM PCB | 1 |
|  | NRE for fabrication of MIL Grade DIOM PCB | 1 |
|  | Fabricated MIL Grade DIOM PCB along with Group-A report and CoC | 8 |
|  | Group-B Report for Fabricated MIL Grade DIOM PCB | 1 |
|  | NRE for fabrication of MIL Grade Backplane PCB | 1 |
|  | Fabricated MIL Grade Backplane PCB along with Group-A report and CoC | 8 |
|  | Group-B Report for Fabricated MIL Grade Backplane PCB | 1 |
|  | NRE for fabrication of MIL Grade SPMC PCB | 1 |
|  | Fabricated MIL Grade SPMC PCB along with Group-A report and CoC | 8 |
|  | Group-B Report for Fabricated MIL Grade SPMC PCB | 1 |
|  | Kit binning Report of Components of IOCM Card as per Annexure-1 along with CoC and Reports | 8 Set |
|  | Components of IOCM Card as per Annexure-1 along with CoC and Reports | 2 set |
|  | Kit binning Report of Components of Backplane Card as per Annexure-2 along with CoC and Reports | 8 Set |
|  | Components of Backplane Card as per Annexure-2 along with CoC and Reports | 2 set |
|  | Kit binning Report of Components of SPMC Card as per Annexure-3 along with CoC and Reports | 8 Set |
|  | Components of SPMC Card as per Annexure-3 along with CoC and Reports | 2 set |
|  | NRE Charges for Assembly of IOCM | 1 |
|  | NRE Charges for Assembly of DIOM | 1 |
|  | NRE Charges for Assembly of Backplane | 1 |
|  | NRE Charges for Assembly of SPMC | 1 |
|  | IOCM Assembly along with Assembly Reports | 8 |
|  | DIOM Assembly along with Assembly Reports | 8 |
|  | Backplane Assembly along with Assembly Reports | 8 |
|  | SPMC Assembly along with Assembly Reports | 8 |
|  | Conformal coating of IOCM with Reports | 8 |
|  | Conformal coating of DIOM with Reports | 8 |
|  | Conformal coating of Backplane with Reports | 8 |
|  | Conformal coating of SPMC with Reports | 8 |
|  | IOCM Rework Jobs along with Reports (including collection of Board from HAL and Delivery of Board to HAL) | 5 |
|  | DIOM Rework Jobs along with Reports (including collection of Board from HAL and Delivery of Board to HAL) | 5 |
|  | Backplane Rework Jobs along with Reports (including collection of Board from HAL and Delivery of Board to HAL) | 5 |
|  | SPMC Rework Jobs along with Reports (including collection of Board from HAL and Delivery of Board to HAL) | 5 |
|  | Batch NRE for Collection of items from HAL and Delivery of Assembled Boards/Conformal Coated/Reworked Boards to HAL | 60 |

Note: Quantity for Batch NRE for Collection of items from HAL and Delivery of Assembled Boards/Conformal Coated Boards to HAL are considered as 20 No’s. Each Type of Board will be provided in maximum of 5 Batches for Assembly. Therefore, maximum of 20 Assembly Batches possible for 4 Type of Boards. Each Type of Board will be provided in maximum of 5 Batches for Conformal Coating. Therefore, maximum of 20 Conformal Coating Batches possible for 4 Type of Boards. Each Type of Board will be provided in maximum of 5 Batches for Rework. Therefore, maximum of 20 Rework Batches possible for 4 Type of Boards. Overall, a maximum 60 Batches are possible and minimum of 3 Batches Possible (if all 4 Types of boards, all quantities provided at a time for assembly and if all 4 Types of boards all, quantities provided at a time for conformal coating / rework). Batch NRE charges will be paid based on number of times vendor collected the Boards from HAL, not based on quantity of Boards and Type of Boards provided by HAL during collection.

- Acceptance

Acceptance shall be provided after delivery of deliverables as mentioned in section 7. In case of reports and documents, if any changes are proposed by HAL, MCSRDC then the vendor shall be ready for incorporating those changes in the reports and documents.

- Repeat Order Requirement

Vendor shall agree for the following items as options:

  - Repeat order for minimum 3No’s. to maximum 8No’s of cards.

  - Repeat Order will be option for HAL, MCSRDC for exercising within 12 months. (The prices, terms and conditions of the repeat order shall be the same as that of the original without NRE Charges).

- Warranty

The vendor shall be ready for rework on the PCBs in case any assembly related issue arises during functional tests within 12 months of delivery.

Annexure-1:

BOM of IOCM

| Sl.No. | Part Number | Description | Manf. | Qty | Remarks/ Temperature |
| --- | --- | --- | --- | --- | --- |
| 1 | 74LVCH16245ADL | 16-bit bus transceiver 5 V tolerant | NXP Semiconductors | 6 | -40°C to +125°C (COTS) |
| 2 | CY7C1041BN-20ZSXA | 256K X 16 Static RAM, Access Time: 20ns 44-Pin TSOP | Cypress Semiconductor | 2 | -40°C to +85°C (COTS) |
| 3 | AM29F160DB-75EI | 1M x16-bi t Bottom Boot code Sector Flash Memory Access Time: 70ns | Spansion | 2 | -40°C to +85°C (COTS) |
| 4 | FM1808B-SG | 256Kb Byte wide FRAM Memory Access Time: 70ns | Ramtron | 2 | -40°C to +85°C (COTS) |
| 5 | MAX705ESA | Reset Generator (Supervisory IC) | Maxim | 2 | -40°C to +85°C (COTS) |
| 6 | SN74LVTH162245DL | 3.3-V ABT 16-Bit Bus Transceivers with 3-State Outputs | Texas Instruments | 6 | -40°C to +85°C (COTS) |
| 7 | MAX3095ESE | Quad RS-422/RS485 Receiver | Maxim | 2 | -40°C to +85°C (COTS) |
| 8 | SP3232EEA-L/TR | +3.0V to +5.5V RS-232 Transceivers | Maxim | 2 | -40°C to +85°C (COTS) |
| 9 | MC68EN360CEM25L | Quad Integrated Communication Controller, Frequency – 25MHz | Freescale Semiconductor | 2 | -40°C to +85°C (COTS) |
| 10 | CXO600LKN001M843200 | 1.8432MHz Clock Crystal Oscillator, 5V, 4 -pin, ±100PPM | Andhra Electronics | 2 | -55°C to +125°C (COTS) |
| 11 | CXO600LKN025M000000 | 25MHz Clock Crystal Oscillator,5V, 4 -pin,±100PPM | Andhra Electronics | 2 | -55°C to +125°C (COTS) |
| 12 | CXO600LKN020M000000 | 20MHz Clock Crystal Oscillator, 5V, 4 -pin,±100PPM | Andhra Electronics | 2 | -55°C to +125°C (COTS) |
| 13 | LXT905LE | Universal 10Base-T Transceiver with 3.3V support | Cortina | 2 | -40°C to +85°C (COTS) |
| 14 | S553-5999-F2 | 10BASE -T LAN Magnetics, Dual Isolation | BEL | 2 | -40°C to +85°C (COTS) |
| 15 | CY7C057V_15AXI | 3.3 V, 32K X 36 FLEx36 Asynchronous Dual-port Static RAM, Access Time: 15ns | Cypress Semiconductor | 2 | -40°C to +85°C (COTS) |
| 16 | MAX3040ESE | Quad RS 422/RS485 Transmitter | Maxim | 2 | -40°C to +85°C (COTS) |
| 17 | PCI9656-BA66BI | 64-Bit, 66 MHz PCI Bus Mastering I/O Accelerator | PLX Technology | 2 | -40°C to +85°C (COTS) |
| 18 | LC4256V-75T176E | Super -FAST High Density PLD 3.3V,176 pin 256 Macrocells 128 I/O options 5V I/O tolerance Tpd =7.5 ns | Lattice Semiconductor | 2 | -40°C to+130°C (COTS) |
| 19 | SN74ALVC164245DL | 16 bit 3.3V/ 5V level shifting transceiver | Texas Instruments | 2 | -40°C to +85°C (COTS) |
| 20 | CXO825LKN-066M000000 | 66MHz Clock Crystal Oscillator 5V,4-pin,±100PPM | Andhra Electronics | 2 | -55°C to +125°C (COTS) |
| 21 | AD7656YSTZ | 250 kSPS, 6 - Channel Simultaneous Sampling Bipolar 16 -Bit ADC | Analog Devices | 2 | −40 °C to +125°C (COTS) |
| 22 | TL074ID | Low Noise JFET-Input operational Amplifier | Texas Instruments | 30 | -40°C to +85°C (COTS) |
| 23 | MAX674ESA+ | 10V Precision Voltage Reference | Maxim | 12 | -40°C to +85°C (COTS) |
| 24 | INA105KU | Precision Unity Gain Differential Amplifier | Burr -Brown | 6 | -40°C to +85°C (COTS) |
| 25 | AD5362BSTZ | 8 -Channel, 16-Bit, Serial Input, Voltage Output DAC | Analog Devices | 2 | -40 to +85°C (COTS) |
| 26 | ADG1219BRJZ-R2 | Low Capacitance, Low Charge Injection,±15 V/12 V CMOS SPDT | Analog Devices | 2 | −40°C to +125°C (COTS) |
| 27 | ADG5404BRUZ | High Voltage, Latch-up Proof, 4 -Channel Multiplexer | Analog Devices | 10 | −40°C to +125°C (COTS) |
| 28 | NX4108CZ1TR_1 | 1A Synchronous Buck Switcher With FET On Board | Microsemi | 2 | -40°C to +85°C (COTS) |
| 29 | ADR4550BRZ | Ultra low noise, high accuracy +5V Voltage Reference | Analog Devices | 2 | -40°C to +125°C (COTS) |
| 30 | 1N4448WS-7-F | Surface Mount Fast Switching Diode Case | Diode Inc. | 2 | -65°C to +150°C (COTS) |
| 31 | BAT754 | Fast switching speed, high conductance surface mount diode | NXP | 4 | -65°C to +125°C (COTS) |
| 32 | XAL4020-102ME | High Temperature Shielded Power Inductors 1uH | Coilcraft | 6 | -40°C to +85°C (COTS) |
| 33 | DO1608C-222 | Inductor 2.2uH 20%, 100Khz | Coilcraft | 2 | -40°C to +85°C (COTS) |
| 34 | CDR31BP471BJURX | Ceramic capacitors 470pF/100V Size: 0805 Tolerance: ±5% TCC: 0 ppm/°C ± 30 ppm/°C Failure Rate: 0.01% | Vishay | 1 | -55°C to +125°C (MIL) |
| 35 | LMK107BJ106MALTD | Ceramic capacitors 10uF/10V Size: 0603 Tolerance: ±20% Dielectric: X5R | Taiyo Yuden | 1 | -55°C to +85°C (COTS) |
| 36 | T491A106K016A | Tantalum Capacitors 10uF/ 16V Case Size: A Tolerance: ±10% | Kemet | 1 | -55°C to +125°C (COTS) |
| 37 | CL10A226MQ8NRNE | Ceramic capacitors 22uF/6.3V Size: 0603 Tolerance: ±20% Dielectric: X5R | Samsung | 1 | -55°C to +85°C (COTS) |
| 38 | CWR16JH476JTHC | Tantalum capacitors 47uF/20V Case size: H, Tolerance: 5% Failure Rate: 0.01%/1000hrs | Vishay | 2 | -55°C to +125°C (MIL) |
| 39 | T97R106K075CBX | Solid tantalum Capacitor 10µF/75V Case Size: R Tolerance:±10% Reliability:0.1% Weibull | Vishay | 2 | -55°C to +125°C (COTS) |
| 40 | T97R226K050CBX | Tantalum capacitor 22uF/50V Case size: R Tolerance: ±10% Reliability:0.1% Weibull | Vishay | 3 | -55°C to +125°C (COTS) |
| 41 | T491D156K035AT | Tantalum Capacitors 1 5 uF/ 35V , Case Size: D Tolerance: ±10% Reliability:0.01% | Kemet | 3 | -55°C to +125°C (COTS) |
| 42 | CDR33-BX-104-A-K-U-S-A-X | Ceramic capacitors 100nF/50V Size: 1210 Tolerance = ±10% TCC: ± 15 % from- 55 °C to +125 °C Failure Rate:0.001% | Vishay | 4 | -55°C to +125°C (MIL) |
| 43 | T495C105K050ATE1K3 | Tantalum capacitor 1uF/50V Case Size: C(6032-28), Tolerance: ±10% Failure Rate: NA | Kemet | 5 | -55°C to +125°C (MIL) |
| 44 | CWR16MH156JTHC | Tantalum capacitor 15uF/35V Case size: H Tolerance:±5% Failure Rate: 0.01%/1000hrs | Vishay | 4 | -55°C to +125°C (MIL) |
| 45 | CWR11NH475JTC ALT: T495D475K050ATE300 | Tantalum Capacitor 4.7uF/50V Case Size: D(7343-31) Industrial Grade, Low ESR Tolerance: ±5% Failure Rate:0.01%/1000hrs | Vishay | 5 | -55°C to +125°C (MIL) |
| 46 | 03028-BR104YJMCB | Ceramic capacitors 100nF/16V Size: 0603 Tolerance : ±5% Testing: Full group C | Vishay | 14 | -55°C to +125°C (COTS) |
| 47 | 08053C105KA72A | Ceramic Capacitors 1uF/ 25V Size:0805 Tolerance: ±10% Dielectric: X7R | AVX | 13 | -55°C to +125°C (COTS) |
| 48 | T419H106K035BC-X | Tantalum Capacitors 1 0 uF/ 35V , Case Size: H Tolerance: ±10% Failure Rate:0.1%/ 1000hrs | Kemet | 15 | -55°C to +125°C (MIL) |
| 49 | 03028-BX-102-B-J-M-C-X | Ceramic capacitor 1nF/100V Size: 0603 Tolerance:5% Testing: Full group C | Vishay | 25 | -55°C to +125°C (COTS) |
| 50 | 03028-BR-103-A-J-M-C-X | Ceramic Capacitors 10nF/50V, Size: 0603 Tolerance: ±5 % Testing: Full group C | Vishay | 30 | -55°C to +125°C (COTS) |
| 51 | 03028BP101BJMCB | Ceramic capacitors 100pF/100V Size: 0603 Tolerance: ±5% Testing: Full group C | Vishay | 34 | -55°C to +125°C (COTS) |
| 52 | 08051C104KAT2A | Ceramic Capacitors 100nF/ 100 V , Size: 0805 Tolerance: ±10% Dielectric: X7R | Vishay | 34 | -55°C to +125°C (COTS) |
| 53 | CDR31BX103AKUSX | Ceramic capacitors 10nF/50V Size = 0805 Tolerance = ±10% TCC: ± 15 % from - 55 °C to + 125 °C Failure Rate:0.001% | Vishay | 84 | -55°C to +125°C (MIL) |
| 54 | D55342-K-07-B-330D-R-TS | Resistor 330E/250mW, Size:1206, Tolerance:1% Failure Rate: 0.01 % per 1000 hrs, TCR:± 100 ppm /°C | Vishay | 1 | -55°C to +125°C (MIL) |
| 55 | M55342-K-06-B-10E0-R-TS | Resistor 10K/ 150 mW, Size:0805, Tolerance:1% Failure Rate: 0.01 % per 1000 h TCR:±100ppm/°C | Vishay | 1 | -55°C to +125°C (MIL) |
| 56 | D55342-K-07-B-7E50-R-TS | Resistor 7.5K/250mW Size:1206 Tolerance: ±1% Failure Rate:0.01 % per 1000 h TCR:±100ppm/°C | Vishay | 1 | -55°C to +125°C (MIL) |
| 57 | CRCW1206332KFK | Resistor 332K/250mW Size:1206 Tolerance: ±1% Failure Rate:0. 3×10-9 / h r TCR:±100ppm/°K | Vishay | 1 | -55°C to +155°C (COTS) |
| 58 | CRCW1206105KFK | Resistor 105K/250mW Size:1206 Tolerance:±1% Failure Rate:0. 3×10-9 /h r TCR:±100ppm/°C | Vishay | 1 | -55°C to +155°C (COTS) |
| 59 | CRCW0603-11R8-F-K | Resistor 11.8E/100mW, Size:0603, Tolerance:1% Failure Rate:0.3×10-9 / h r TCR: ±100ppm/K | Vishay | 2 | -55°C to +155°C (COTS) |
| 60 | D55342-K-07-B-20E0-R-TS | Resistor 20K/ 2 50mW Size: 1206 , Tolerance:±1% Failure Rate: 0.01% per 1000 h TCR:±100ppm/°C | Vishay | 2 | -55°C to +125°C (MIL) |
| 61 | D55342-K-07-B-100E-R-TS | Resistor 100K/250mW,Size:1206, Tolerance: ±1% Failure Rate:0.01% per 1000 h TCR:±100ppm/°C | Vishay | 2 | -55°C to +125°C (MIL) |
| 62 | D55342-K-07-B-100D-R-XX | Resistor 100E/ 250 mW Size:1206 Tolerance:±1% Failure Rate: 0.01% per 1000 h TCR:±100ppm/°C | Vishay | 4 | -55°C to +125°C (MIL) |
| 63 | D55342-K-07-B-221D-R-TS | Resistor 220E/250 mW Size:1206 Tolerance:±1% Failure Rate: 0.01 % per 1000 h TCR:± 25ppm/°C | Vishay | 6 | -55°C to +125°C (MIL) |
| 64 | CRCW0805-0000-Z-S-TS | Resistor 0E Size:0805 | Vishay | 11 | -55°C to +155°C (COTS) |
| 65 | CRCW01206-0000-Z-0 | Resistor 0E Size: 1206 | Vishay | 23 | -55°C to +155°C (COTS) |
| 66 | M55342-K-11-B-5E10-R-TS | Resistor 5 .1 K/ 50mW Size:0402 Tolerance:±1% Failure Rate: 0.01% per 1000 h TCR:±100ppm/°C | Vishay | 4 | -55°C to +125°C (MIL) |
| 67 | M55342-K-06-B-100D-R-TS | Resistor 100E/150mW Size:0705 Tolerance:1% Failure Rate: 0.01 % per 1000 h TCR:±100ppm/°C | Vishay | 11 | -55°C to +125°C (MIL) |
| 68 | D55342-K-07-B-1E00-R-TS | Resistor 1K/250mW Size:1206 Tolerance:±1% Failure Rate: 0.01%/1000 hrs TCR:±100ppm/°C | Vishay | 24 | -55°C to +125°C (MIL) |
| 69 | M55342-K-06-B-200E-R-TS | Resistor 200 K/ 150mW Size:0705 Tolerance: ±1% Failure Rate: 0.01 % per 1000 hrs, TCR:±100ppm/°C | Vishay | 26 | -55°C to +125°C (MIL) |
| 70 | M55342-K-12-B-100E-R-TS | Resistor 100K/100mW Size:0603 Tolerance: ±1% Failure Rate: 0.01% per 1000 h TCR:±100ppm/°C | Vishay | 26 | -55°C to +125°C (MIL) |
| 71 | CRCW0603-115K-FKEA | Resistor 115K/100mW Size:0603 Tolerance:±0.1% Failure Rate: 0.01% per 1000 h TCR:±100ppm/°C | Vishay | 27 | -55°C to +155°C (COTS) |
| 72 | M8340109K1002FCD03 (Vishay) Alt: 4610X101103LF (Bourns) | 10-pin Resistor Network 10K/1.08W Tolerance=±1% MIL-PRF-83401 Qualified, Bussed Schematic MSM10A01; 100ppm , 50VDC | SIP | 35 | -55°C to +125°C (MIL) |
| 73 | D55342-K-07-B-10E0-R-TS | Resistor 10K/250mW,Size: 1206, Tolerance:±1%Failure Rate:0.01 % per 1000 h TCR:±100ppm/°C | Vishay | 66 | -55°C to +125°C (MIL) |
| 74 | M55342-K-11-B-4E75-R-TS | Resistor 4.75K/50mW Size:0402 ,Tolerance:1% Failure Rate: 0.01% per 1000 h TCR:±100ppm/°C | Vishay | 78 | -55°C to +125°C (MIL) |
| 75 | CRCW040210K0FK | Resistor 10K /63mW Size: 0402,Tolerance: ±1% Failure Rate:0. 3×10-9 / hr TCR:±100ppm/°C | Vishay | 93 | -55°C to +155°C (MIL) |
| 76 | CRCW0603-10R0-F-K | Resistor 10E/100mW ,Size:0603 Tolerance:±1% Failure Rate:0. 3×10-9 / h r TCR: ±100ppm/K | Vishay | 95 | -55°C to +155°C (COTS) |
| 77 | ERJ -2GE0R00X | Resistor 0E Size: 0402 | Panasonic | 4 | -55°C to +155°C (COTS) |
| 78 | Intentionally Blank |  |  |  |  |
| 79 | 354142 | CPCI Female Connector, Type A without Peg, with shield 110 signal pins | ERNI | 2 | -55°C to +125°C (COTS) |
| 80 | 354148 | CPCI Female Connector, Type B22, with shield 110 signal pins | ERNI | 2 | -55°C to +125°C (COTS) |
| 81 | 354146 | CPCI Female Connector, Type B19, with shield 95 signal pins | ERNI | 1 | -55°C to +125°C (COTS) |
| 82 | F109-3L-BLK EXTRACTOR | Left Card Extractor Al . Alloy Type 5052-H32 , For Board Thickness: 2.4 mm, Black Anodised | Calmark | 1 | - |
| 83 | F109-3R-BLK EXTRACTOR | Right Card Extractor For Board Thickness: 2.4mm Black Anodised | Calmark | 1 | - |
| 84 | CALMARK MVA226-4.80ETM2LK | Card Lock 4.8” length M2.5 HEX Drive, Black Anodized, Wedge Lock Type | Calmark/ Wakefield | 2 | MIL-A-8625 Type II, Class 2 |
| 85 | 8614-00-16-2-Y0-2-317-LF | header_2x8 pin, 6.0mm Contact side Dimension 2.54mm Pitch; | FCI | 2 | -40°C to +100°C (COTS) |
| 86 | 8614-00-10-2-Y0-2-317-LF | header_2x5 pin,6.0mm Contact side dimension, 2.54mm Pitch | FCI | 4 | -40°C to +100°C (COTS) |
| 87 | BN 661 1212028 | M2x 4 CSK Star HD Screw | LPS BOSSARD | 6 | ---- |

Note:

- Active Components: Sl.No.  2, 3, 9, 13, 14, 15, 17, 18 and 28 may be purchased from stockiest and get solderability report and AS6081 test report in case of obsolescence or alternate part with OEM CoC may be suggested as per description along with datasheet (It should be pin-pin compatible and form fit functional)

- Passive Components: Sl.No. 34, 38, 39, 40, 42, 44, 46 to 51, 53 to 58, 60 to 63, 65, 66, 67, 69, 73, 79, 80 and 81 may be replaced with alternate part with OEM CoC as per description in case of non-availability and obsolescence. CRCW series resistors can also be checked in place of D55342/M55342 series.

Annexure-2:

BOM of Backplane

| S/N | Part Reference | Part Number | Manf. | Qty | Remarks/Temperature |
| --- | --- | --- | --- | --- | --- |
| 1 | C1 connector, Wall mount receptacle Aluminium shell, olive drab cadmium over electroless nickel base Mil Grade 6 pins circular connector_ series III, N keying with minimum 5mm PCB solder tail | D38999/20WB98PN-XX-YYY | Conesys/ Amphenol/ RayQ/ RF Immunity/ Equivalent | 1 | XX INDICATES PIN DIA (̴̴ 0.8mm) & YYY (8mm) INDICATES PIN LENGTH for PCB Solder Pins customised by OEM Operating Temp. -65 °C to +175 °C (MIL) |
| 2 | C2 connector, Wall mount receptacle Aluminium shell, olive drab cadmium over electroless nickel base Mil grade circular connector_55 pins series III, A keying with minimum 5mm PCB solder tail | D38999/20WE35PA- XX-YYY | Conesys/ Amphenol/ RayQ/ RF Immunity/ Equivalent | 1 | XX INDICATES PIN DIA (̴̴ 0.8mm) & YYY (8mm) INDICATES PIN LENGTH for PCB Solder Pins customised by OEM Operating Temp. -65 °C to +175 °C (MIL) |
| 3 | C3 connector, Wall mount receptacle Aluminium shell, olive drab cadmium over electroless nickel base Mil grade circular connector_128 pins series III, N keying with minimum 5mm PCB solder tail | D38999/20WJ35PN- XX-YYY | Conesys/ Amphenol/ RayQ/ RF Immunity/ Equivalent | 1 | XX INDICATES PIN DIA (̴̴ 0.8mm) & YYY (8mm) INDICATES PIN LENGTH for PCB Solder Pins customised by OEM Operating Temp. -65 °C to +175 °C (MIL) |
| 4 | C4 connector, Wall mount receptacle Aluminium shell, olive drab cadmium over electroless nickel base Mil grade circular connector_55 pins series III, N keying with minimum 5mm PCB solder tail | D38999/20WE35PN- XX-YYY | Conesys/ Amphenol/ RayQ/ RF Immunity/ Equivalent | 1 | XX INDICATES PIN DIA (̴̴ 0.8mm) & YYY (8mm) INDICATES PIN LENGTH for PCB Solder Pins customised by OEM Operating Temp. -65 °C to +175 °C (MIL) |
| 5 | C5 connector, Wall mount receptacle Aluminium shell, olive drab cadmium over electroless nickel base Mil grade circular connector_128 pins series III, A keying with minimum 5mm PCB solder tail | D38999/20WJ35PA- XX-YYY | Conesys/ Amphenol/ RayQ/ RF Immunity/ Equivalent | 1 | XX INDICATES PIN DIA (̴̴ 0.8mm) & YYY (8mm) INDICATES PIN LENGTH for PCB Solder Pins customised by OEM Operating Temp. -65 °C to +175 °C (MIL) |
| 6 | C6 connector, Wall mount receptacle Aluminium shell, olive drab cadmium over electroless nickel base Mil grade circular connector_128 pins series III, B keying with minimum 5mm PCB solder tail | D38999/20WJ35PB- XX-YYY | Conesys/ Amphenol/ RayQ/ RF Immunity/ Equivalent | 1 | XX INDICATES PIN DIA (̴̴ 0.8mm) & YYY (8mm) INDICATES PIN LENGTH for PCB Solder Pins customised by OEM Operating Temp. -65 °C to +175 °C (MIL) |
| 7 | Compact PCI (P1, P4) Press-fit connector, Type A with peg, (22*7)=154 Pins, Pin Pitch 2mm, PICMG 2.3 Rev 3.0 | 923190 Alt 17011542203* | ERNI Alt HARTING | 8 | Operating Temp. -55 °c to +125 °c (COTS) |
| 8 | Compact PCI (P2, P5) Press-fit connector, Type B, (22*7)=154 Pins, Pin Pitch 2mm, PICMG 2.3 Rev 3.0 | 914796 Alt 17041542203* | ERNI Alt HARTING | 8 | Operating Temp. -55 °c to +125 °c (COTS) |
| 9 | Compact PCI (P3) Press-fit connector, Type B, (19*7)=133 Pins, Pin Pitch 2mm, PICMG 2.3 Rev 3.0 | 103670 Alt 17051332203* | ERNI Alt HARTING | 4 | Operating Temp. -55 °c to +125 °c (COTS) |
| 10 | Receptacle Type:4 row-300 pin connector (.075”), Socket Straight Dip Solder (Female), Gold Plated with full round guide pin (tail length 0.172 inch) | RM422-300-831-9100 | Airborn | 1 | Operating Temp. -65 °c to +125 °c (MIL-C-55302) |
| 11 | Receptacle Type: 3 row-122 pin connector (.075”), Socket Straight Dip Solder (Female) ,Gold Plated with full round guide pin (tail length 0.172 inch) | RM322-122-221-5570 | Airborn | 2 | Operating Temp. -65 °c to +125 °c (MIL-C-55302) |
| 12 | Receptacle Type: 2 row-010 pin connector (.075”), Socket Straight Dip Solder (Female), Gold Plated with full round guide pin (tail length 0.172 inch). | RM222-010-221-2900 | Airborn | 1 | Operating Temp. -65 °c to +125 °c (MIL-C-55302) |
| 13 | Vertical Receptacle assembly, short guides, Multi Beam-XLE VITA 62 Standard J0 | 6450863-5 | TE connectivity | 1 | Operating Temp. -40 °c to +125 °c (COTS) |
| 14 | Vertical Receptacle assembly, W/LP contacts, without guides, Multi Beam-XLE VITA 62 Standard J1 | 1-6450869-0 | TE connectivity | 1 | Operating Temp. -40 °c to +125 °c (COTS) |
| 15 | Capacitor | 03028-BR103AJMC | Vishay | 4 | Operating Temp. -55 °c to +125 °c (COTS) |
| 16 | Capacitor | T491D156K035AT | KEMET | 8 | Operating Temp. -55 °c to +125 °c (COTS) |
| 17 | Capacitor | T491D476K025AT | KEMET | 16 | Operating Temp. -55 °c to +125 °c (COTS) |
| 18 | RESISTOR, 4.75K, 1%, 0805, 100 PPM | M0805K4751FBT0 | Vishay | 3 | Operating Temp. -55 °c to +125 °c (COTS) |
| 19 | Resistor | CRCW0805-0000-Z-0 | Vishay | 40 | Operating Temp. -55 °c to +125 °c (COTS) |
| 20 | Intentionally blank |  |  |  |  |
| 21 | Common mode Choke , 120±25% μH Voltage Rating >100 Vac,Current Rating >15A, 4 Pin Through Hole | MT-8828 | Magna tech | 1 | Operating Temp. -54°C to+85°C (COTS) |
| 22 | Capacitor | CKR05BX102KR* Alternate Part No: SR212C102KA600C | AVX Alternate Ploy-AVX | 2 | Operating Temp. -55°C to 125°C |
| 23 | Capacitor | CKR05BX152KR* Alternate Part No: SR212C152KA600C | AVX Alternate Ploy-AVX | 2 | Operating Temp. -55°C to 125°C |
| 24 | EMI Filter Min 3000 pF, Pi- Type, 100VDC/15, 3 Pin Through Hole, | CS-5000-18 | Cera Seals, Bangalore | 2 | Operating Temp. -55°C to+125°C (COTS) |
| 25 | Spacer Hex Standoff M3 Brass 8mm | R30-1000802 | MPN Harwin | 24 | -- |
| 26 | Screw Pan Cross Recessed SST M3x6 | 1250477 | LPS Bossard | 24 | -- |
| 27 | Screw CSK Cross Recessed SST M3x5 | 1250469 | LPS Bossard | 24 | -- |
| 28 | 2.0 mm ERmet Hard Metric Connector Coding Key, Strawberry Red used in for S1P1 & S1P4 connector | 43350 | ERNI | 2 | Operating Temp. -55 °c to +125 °c (COTS) |
| 29 | 2.0 mm ERmet Hard Metric Connector Coding Key, Brilliant Blue used in for S2P1 & S2P4 connector | 43347 | ERNI | 2 | Operating Temp. -55 °c to +125 °c (COTS) |
| 30 | 2.0 mm ERmet Hard Metric Connector Coding Key ,Cadmium Yellow used in for S3P1 & S3P4 connector | 43345 | ERNI | 2 | Operating Temp. -55 °c to +125 °c (COTS) |
| 31 | 2.0 mm ERmet Hard Metric Connector Coding Key, Resenda Green used in for S4P1 & S4P4 connector | 43346 | ERNI | 2 | Operating Temp. -55 °c to +125 °c (COTS) |
| 32 | M1 connector, Plug type with socket, Aluminium shell, olive drab cadmium over electroless nickel base Mil Grade 6 pins circular connector_ series III, N keying | D38999/26WB98SN | Conesys/ Amphenol/ RayQ/ RF Immunity/ Equivalent | 1 | Operating Temp. -65 °C to +175 °C (MIL) |
| 33 | M2 connector, Plug type with socket, Aluminium shell, olive drab cadmium over electroless nickel base Mil grade circular connector_55 pins series III, A keying | D38999/26WE35SA | Conesys/ Amphenol/ RayQ/ RF Immunity/ Equivalent | 1 | Operating Temp. -65 °C to +175 °C (MIL) |
| 34 | M3 connector, Plug type with socket, Aluminium shell, olive drab cadmium over electroless nickel base Mil grade circular connector_128 pins series III, N keying | D38999/26WJ35SN | Conesys/ Amphenol/ RayQ/ RF Immunity/ Equivalent | 1 | Operating Temp. -65 °C to +175 °C (MIL) |
| 35 | M4 connector, Plug type with socket, Aluminium shell, olive drab cadmium over electroless nickel base Mil grade circular connector_55 pins series III, N keying | D38999/26WE35SN | Conesys/ Amphenol/ RayQ/ RF Immunity/ Equivalent | 1 | Operating Temp. -65 °C to +175 °C (MIL) |
| 36 | M5 connector, Plug type with socket, Aluminium shell, olive drab cadmium over electroless nickel base Mil grade circular connector_128 pins series III, A keying | D38999/26WJ35SA | Conesys/ Amphenol/ RayQ/ RF Immunity/ Equivalent | 1 | Operating Temp. -65 °C to +175 °C (MIL) |
| 37 | M6 connector, Plug type with socket, Aluminium shell, olive drab cadmium over electroless nickel base Mil grade circular connector_128 pins series III, B keying | D38999/26WJ35SB | Conesys/ Amphenol/ RayQ/ RF Immunity/ Equivalent | 1 | Operating Temp. -65 °C to +175 °C (MIL) |
| 38 | -- | M85049/39-17N | Conesys/ Amphenol/ RayQ/ RF Immunity/ Equivalent | 1 | Operating Temp. -65 °C to +175 °C (MIL) |
| 39 | -- | M85049/39-25N | Conesys/ Amphenol/ RayQ/ RF Immunity/ Equivalent | 2 | Operating Temp. -65 °C to +175 °C (MIL) |

Note:

- Sl.No.  18, 21, 24, 28, 29, 30 and 31 can be replaced with alternate part with OEM CoC as per description in case of obsolescence and non-availability .

- Item Listed at Sl.No. 32 to 39 for 8 sets shall be delivered to HAL along with 2 Set spare components

- Sl.No. 24 can be explored through Cera Seals, Bangalore.

- Sl.No. 21 can be explored through Magna Tech, Bangalore or alternate can be procured. Specification of Sl.No. 21 are as follows:

KEY PARAMETERS

    - Core Material 		          :  Ferrite Toroid

    - Inductance 		          :  As per Table-1

    - Voltage Rating 		          :  ≥100V

    - Current Rating 		          :  ≥15A

    - DC Resistance 		          :  ≤ 10milli ohm per each winding

    - Mounting Style 		          :  Horizontal, with Solderable Leads as shown in Fig 1

    - Operating Temperature           :  -54⁰C to +85⁰C

    - Height (HT) (Ref fig-1)            : 12mm ±2mm

    - Outer Diameter (OD) (Ref fig-1)  :  ≤ 30mm

Figure – 1

Annexure-3:

BOM of SPMC

| S/N | Part Number | Description | Manuf. | Qty | Temperature |
| --- | --- | --- | --- | --- | --- |
| 1 | TPS72501DT | IC | TI | 3 | −40°C to +125°C (COTS) |
| 2 | THS4130IDGK | IC | TI | 6 | –40°C to +85°C (COTS) |
| 3 | AM26LS31MFKB | IC | TI | 2 | –55°C to +125°C (COTS) |
| 4 | APA300BG456I | IC | Actel | 2 | –40°C to +85°C (COTS) |
| 5 | SN74AVC2T245RSWR | IC | TI | 2 | –40°C to +85°C (COTS) |
| 6 | AM26LS32AMFKB | IC | TI | 2 | –55°C to +125°C |
| 7 | CY2304NZZI-1 | IC | Cypress | 2 | –40°C to +85°C (COTS) |
| 8 | PCI9030-AA60PI F | IC | PLX Technology | 2 | –40°C to +85°C (COTS) |
| 9 | CY7C057V-15AXI | IC | Cypress | 6 | –40°C to +85°C (COTS) |
| 10 | DAC5672AIPFBG4 | IC | TI | 3 | –40°C to +85°C (COTS) |
| 11 | CXO-7050-I-M-3-N-050M000000 | Crystal oscillator | ANDRA ELECTRONICS | 2 | –40°C to +85°C (COTS) |
| 12 | 71436-2864 | Connector | Molex | 4 | -55°C to +85°C (COTS) |
| 13 | CRCW0603-0000-Z-0 | Resistor | VISHAY | 32 | –55°C to +155°C (COTS) |
| 14 | CRCW0603-33R0-F-K-TA | Resistor | VISHAY | 5 | –55°C to +155°C (COTS) |
| 15 | CRCW0402-100R-F-K-TA | Resistor | VISHAY | 40 | –55°C to +155°C (COTS) |
| 16 | CRCW0603-100R-F-K-TA | Resistor | VISHAY | 9 | –55°C to +155°C (COTS) |
| 17 | TNPW060360R0BY | 60E, 100mW, Thin film Resistor, size=0603, tolerance=±0.1%, ppm=±10 | VISHAY | 8 | –55°C to +125°C (COTS) |
| 18 | TNPW0402300RBY | 300E, 63mW, Thin film Resistor, size=0402, tolerance=±0.1%, ppm=±10 | VISHAY | 8 | –55°C to +125°C (COTS) |
| 19 | TNPW06031K92BY | 1.92K, 100mW, Thin film Resistor, size=0603, tolerance=±0.1%, ppm=±10 | VISHAY | 4 | –55°C to +125°C (COTS) |
| 20 | CRCW0603-4K70-F-K-TA | Resistor | VISHAY | 4 | –55°C to +155°C (COTS) |
| 21 | CRCW0402-4K70-F-K-TA | 4.7K, 63mW, Thick film Resistor, size=0402, tolerance=±1%, ppm=±100, | VISHAY | 24 | –55°C to +155°C (COTS) |
| 22 | CRCW0603-5K10-F-K-TA | Resistor | VISHAY | 3 | –55°C to +155°C (COTS) |
| 23 | CRCW0603-10K0-F-K-TA | Resistor | VISHAY | 9 | –55°C to +155°C (COTS) |
| 24 | CRCW0603-115K-F-K-TA | Resistor | VISHAY | 1 | –55°C to +155°C (COTS) |
| 25 | CRCW0603-121K-F-K-TA | Resistor | VISHAY | 1 | –55°C to +155°C (COTS) |
| 26 | CRCW0603-127K-F-K-TA | Resistor | VISHAY | 1 | –55°C to +155°C (COTS) |
| 27 | CRCW0603-196K-F-K-TA | Resistor | VISHAY | 1 | –55°C to +155°C (COTS) |
| 28 | TNPW06033K00BY | 3K, 100mW, Thin film Resistor, size=0603, tolerance= ±0.1%, ppm=±10 | VISHAY | 8 | –55°C to +125°C (COTS) |
| 29 | 0603-3-D-104-K-A-T-2-A | Capacitor | AVX | 8 | -55°C to +85°C (COTS) |
| 30 | 0402-Y-D-104-K-A-T-2-A | Capacitor | AVX | 110 | -55°C to +85°C (COTS) |
| 31 | VJ0603-Y-475-J-X-J-C | 4.7uF/16V ceramic capacitor, X7R size=0603, tolerance=±5%, -55°C to +125°C | VISHAY | 2 | -55°C to +125°C (COTS) |
| 32 | 1206-3-D-106-K-A-T-2-A | Capacitor | AVX | 16 | -55°C to +85°C (COTS) |
| 33 | 1206-Y-D-226-K-A-T-2-A | Capacitor | AVX | 10 | -55°C to +85°C (COTS) |
| 34 | 1206-6-D-476-K-A-T-2-A | Capacitor | AVX | 3 | -55°C to +85°C (COTS) |
| 35 | Intentionally Blank | -- | -- | -- | -- |
| 36 | Intentionally Blank | -- | -- | -- | -- |
| 37 | 93LC66B-I/ST | IC | Microchip | 1 | -40°C to +85°C (COTS) |
| 38 | AM26LS31MFKB | IC | TI | 1 | -55°C to +125°C (COTS) |
| 39 | 90120-0127 | Connector | Molex | 1 | -55°C to +125°C (COTS) |
| 40 | FTSH-113-01-L-DV-K | Connector | Samtec | 1 | -55°C to +125°C (COTS) |

Note:

- Active Components: Sl.No.  1, 2, 4, 7 and 8 can be purchased from stockiest and get solderability report and As6081 test report in case of obsolescence or alternate part with OEM CoC may be suggested as per description along with datasheet (It should be pin-pin compatible and form fit functional)

- Passive Components: Sl.No.  17, 18, 19, 21, 28, 31 and 34 can be replaced with alternate part as per description in case of obsolescence or non-availability.

Annexure-4:

Dimension of Backplane

30 mm

146mm mm

264 mm

Flexi

Slot1 (cPCI Connector- Press  Fit)

cPCI BUS

C4

C3

C1

Slot2 (cPCI Connector- Press  Fit)

Flexi

Slot3 (cPCI Connector- Press  Fit)

146mm mm

146mm mm

Slot4 (cPCI Connector- Press  Fit)

Local Bus

Flexi

C5

C2

Slot6 (3Row-120 Pin)

Slot6 (3Row-120 Pin)

Slot5 (4 Row - 300 Pin connector)

FANCON

ATX

Flexi

Slot 7 (Power Supply connector VITA-62)

C6

Fig - Dimension Details of Backplane & Connector Block PCB with Flexi